- How to use online for mobile DVR

- 1. First you need to install the IVMS Server, then add a account and device in the web side.

- Login the MDVR with default password 888888, then choose ‘BASIC’--

- ’VEHICLE’, change the Device No. to the device you add in web side, save it.

- 3. Go back to ‘BASIC’, choose "NETWORK", change the server IP and port to your server's WAN IP and port, then save it.

- 4. Go back to the setup menu, then choose 'PREIPHERALS'--'CELLULAR' , change the 4G parameters, you can ask your local 4G operators for those information: APN, Center Num, User Name, Password. After you fill out this, save it.

- 5. Go back to the Video preview window, click the info button on the remote control, you can see the system info page, wait a few mins, it will show PLATFORM SUCCESSFUL, this means the MDVR is online, then you can check the video in IVMS Client side.